Scatterplot: BMR vs bathymetry, coloured by ice draft depth. To incorporate grounding line proximity into my scatterplot as a marker shape:

CREATING A PROXIMITY RASTER:

- Load/create a polyline shapefile which traces the grounding line

- Rasterise the grounding line

  - Click Rasterise (vector to raster): this creates a raster for the grounding line shapefile

    - input layer: grounding line shapefile

    - burn in value field: set to 1

    - output raster size units: pixels?

    - Set width and height to match reference raster

  - Compute proximity: this creates a raster which calculates the distance to the GL shapefile

    - In processing toolbox, search Proximity (Raster Distance)

      - Input raster is the rasterised grounding line

      - Values to calculate distances for: 1

After proximity raster is calculated and exported, add path and run the following code:

import rasterio import numpy as np import matplotlib.pyplot as plt import matplotlib.lines as mlines # Load rasters with rasterio.open("WAOM_2011_BMR.tif") as src1, \ rasterio.open("WAOM_2011_h_bathymetry.tif") as src2, \ rasterio.open("WAOM_2011_zice_ice_draft.tif") as src3, \ rasterio.open("draft_proximity_map_to_grounding_line.tif") as src4: # Grounding line proximity z1 = src1.read(1) # BMR z2 = src2.read(1) # Bathymetry z3 = src3.read(1) # Ice draft z4 = src4.read(1) # Distance to grounding line nodata1, nodata2, nodata3, nodata4 = src1.nodata, src2.nodata, src3.nodata, src4.nodata # Create mask mask = (~np.isnan(z1)) & (~np.isnan(z2)) & (~np.isnan(z3)) & (~np.isnan(z4)) if nodata1 is not None: mask &= (z1 != nodata1) if nodata2 is not None: mask &= (z2 != nodata2) if nodata3 is not None: mask &= (z3 != nodata3) if nodata4 is not None: mask &= (z4 != nodata4) mask &= (z1 != 0) # Apply mask z1_valid = z1[mask].flatten() * (365 * 24 * 60 * 60) # BMR in m/yr z2_valid = z2[mask].flatten() # Bathymetry z3_valid = np.abs(z3[mask].flatten()) # Ice draft depth z4_valid = z4[mask].flatten() # Grounding line proximity # Bin grounding line distances into categories bins = [0, 2, 5, 30, np.inf] # in meters bin_labels = ["<2 pixels", "2-5 pixels", "5-30 pixels", ">30 pixels"] shapes = ['o', 's', '^', 'x'] # circle, square, triangle, x bin_indices = np.digitize(z4_valid, bins) - 1 # Plot #plt.figure(figsize=(160/25.4, 100/25.4)) plt.figure(figsize=(12, 8)) for i in range(len(bins) - 1): idx = bin_indices == i plt.scatter(z1_valid[idx], z2_valid[idx], c=z3_valid[idx], cmap='viridis', s=20, marker=shapes[i], label=bin_labels[i]) # Labels and colorbar plt.xlabel("BMR (m/yr)", fontsize=12, fontfamily='Times New Roman') plt.ylabel("Bathymetry (m)", fontsize=12, fontfamily='Times New Roman') plt.tick_params(direction='in') plt.xticks(fontsize=12, fontfamily='Times New Roman') plt.yticks(fontsize=12, fontfamily='Times New Roman') cbar = plt.colorbar() cbar.set_label("Ice Draft Depth (m)", fontsize=12, fontfamily='Times New Roman') cbar.ax.tick_params(labelsize=12) for tick in cbar.ax.get_yticklabels(): tick.set_fontfamily('Times New Roman') legend_handles = [ mlines.Line2D([], [], color='black', marker=shapes[i], linestyle='None', markersize=6, label=bin_labels[i]) for i in range(len(bin_labels)) ] plt.legend(handles=legend_handles, title="Distance to GL", fontsize=10, loc='lower right') plt.tight_layout() # plt.savefig('WAOM_scatter_BMR_Bathymetry_Thickness_proximity_to_GL.jpg', dpi=500, bbox_inches='tight') plt.show()

- The red shapefile is the GL shapefile

- The green pixels are the rasterised versions of the red GL